HYBRID CLOUD ARCHITECTURE

AWS Site-to-Site VPN Lab

Step-by-Step Implementation Guide with Issues & Resolutions

| Detail | Value |
| --- | --- |
| Lab Environment | Ubuntu 24.04 LTS (VMware Workstation) |
| VPN Technology | strongSwan 5.9.13 — IPSec IKEv1 |
| AWS VPN ID | vpn-0a9aa2a54ab7106e3 |
| On-Prem Network | 192.168.91.0/24 |
| Ubuntu VM IP | 192.168.91.145 |
| Router Public IP | 172.220.62.45 (Customer Gateway) |
| AWS VPC | 10.0.0.0/16 |
| Tunnel 1 AWS EP | 52.55.138.58 |
| Tunnel 2 AWS EP | 52.206.45.187 |
| Lab Date | April 7, 2026 |
| Final Status | ✅ FULLY OPERATIONAL — Both tunnels UP |

# Table of Contents

# 1. Lab Overview

This document is a complete record of the AWS Hybrid Cloud Architecture lab — a production-grade Site-to-Site VPN connecting an Ubuntu 22/24 VM running on VMware Workstation to an AWS VPC. The lab covers all AWS infrastructure creation, strongSwan IPSec configuration, VTI interface setup, and full end-to-end connectivity verification.

The document also captures every issue encountered during the lab with the exact error messages, root cause analysis, and the precise commands used to resolve each problem. This makes it a practical troubleshooting reference for anyone implementing the same architecture.

## 1.1 Architecture Summary

| Component | On-Premises | AWS |
| --- | --- | --- |
| Network CIDR | 192.168.91.0/24 | 10.0.0.0/16 |
| VPN Gateway | RRAS / strongSwan on Ubuntu | Virtual Private Gateway (VGW) |
| VPN Protocol | IKEv1 IPSec AES-128 SHA-1 | IKEv1 IPSec AES-128 SHA-1 |
| Authentication | Pre-Shared Key (PSK) | Pre-Shared Key (PSK) |
| Tunnel 1 | 192.168.91.145 → 52.55.138.58 | 169.254.46.237/30 — 169.254.46.238/30 |
| Tunnel 2 | 192.168.91.145 → 52.206.45.187 | 169.254.54.153/30 — 169.254.54.154/30 |
| EC2 Instance | N/A | 10.0.1.23 (Amazon Linux 2023, t3.micro) |

## 1.2 Network Diagram

The following table summarises the full network path from the on-premises Ubuntu VM through the encrypted IPSec tunnel to the AWS private subnet:

| Hop | Device / Service | IP Address | Protocol |
| --- | --- | --- | --- |
| 1 | Ubuntu VM (VMware — Bridged) | 192.168.91.145 | LAN |
| 2 | Home Router (NAT) | 192.168.91.1 → 172.220.62.45 | Internet |
| 3 | IPSec Tunnel (Encrypted) | 172.220.62.45 ↔ 52.55.138.58 | IKEv1 ESP UDP 4500 |
| 4 | AWS Virtual Private Gateway | 52.55.138.58 | AWS backbone |
| 5 | AWS Private Subnet 10.0.1.0/24 | 10.0.1.x | VPC routing |
| 6 | EC2 Instance (Amazon Linux 2023) | 10.0.1.23 | Destination |

📌 Lab Origin This lab was originally planned with Windows Server 2019 as the on-prem VPN gateway using RRAS. Midway through the lab it was switched to Ubuntu 24.04 with strongSwan — which is more reliable behind NAT and better suited to AWS IKEv1 IPSec tunnels.

# 2. Phase 1 — AWS Infrastructure Setup

All AWS resources are created via the AWS Management Console. The following sections walk through each resource in the order they must be created.

## 2.1 Create the VPC

- Navigate to AWS Console → VPC → Your VPCs → Create VPC

- Select VPC only (not VPC and more)

- Set Name tag: hybrid-vpc

- Set IPv4 CIDR block: 10.0.0.0/16

- Leave Tenancy as Default

- Click Create VPC

⚠️ CIDR Overlap Warning The VPC CIDR (10.0.0.0/16) must NEVER overlap with the on-prem CIDR (192.168.91.0/24). Overlapping CIDRs silently break all VPN routing with no error message.

## 2.2 Create Subnets

Navigate to VPC → Subnets → Create Subnet. Select hybrid-vpc and create all three subnets using Add new subnet:

| Subnet Name | Availability Zone | CIDR Block | Purpose |
| --- | --- | --- | --- |
| hybrid-public-1a | us-east-1a | 10.0.0.0/24 | ALB, NAT Gateway, management |
| hybrid-private-1a | us-east-1a | 10.0.1.0/24 | EC2 instances (primary AZ) |
| hybrid-private-1b | us-east-1b | 10.0.2.0/24 | EC2 instances (secondary AZ) |

- After creating — select hybrid-public-1a → Actions → Edit subnet settings

- Enable auto-assign public IPv4 address → Save

## 2.3 Internet Gateway

- VPC → Internet Gateways → Create Internet Gateway → Name: hybrid-igw → Create

- Select it → Actions → Attach to VPC → hybrid-vpc → Attach

## 2.4 Route Tables

### Public Route Table

- VPC → Route Tables → Create Route Table → Name: hybrid-public-rt → VPC: hybrid-vpc

- Routes tab → Edit routes → Add route: 0.0.0.0/0 → Target: hybrid-igw

- Subnet Associations → Edit → check hybrid-public-1a → Save

### Private Route Table

- Create Route Table → Name: hybrid-private-rt → VPC: hybrid-vpc

- Subnet Associations → Edit → check hybrid-private-1a AND hybrid-private-1b → Save

- Do NOT add 0.0.0.0/0 to the private table — the on-prem route will appear via propagation

## 2.5 Virtual Private Gateway (VGW)

The VGW is the AWS-side VPN endpoint. It terminates the IPSec tunnel from the Ubuntu VM.

- VPC → Virtual Private Gateways → Create Virtual Private Gateway

- Name: hybrid-vgw | ASN: Amazon default (64512)

- Create → Select it → Actions → Attach to VPC → hybrid-vpc → Attach

- Wait ~1 minute until state shows Attached

## 2.6 Customer Gateway (CGW)

The CGW represents the on-prem VPN device in AWS. The Ubuntu VM sits behind a NAT router, so the CGW IP is the router's public IP, not the VM's LAN IP.

- VPC → Customer Gateways → Create Customer Gateway

- Name: ubuntu-strongswan | Routing: Static

- IP Address: 172.220.62.45 (router public IP — NOT the Ubuntu VM LAN IP)

- Leave BGP ASN blank → Create Customer Gateway

📌 Why Use the Router IP? The Ubuntu VM (192.168.91.145) sits behind NAT. AWS sees connections arriving from 172.220.62.45. The CGW must use this public IP. strongSwan handles NAT Traversal (NAT-T) via UDP 4500 automatically.

## 2.7 Site-to-Site VPN Connection

- VPC → Site-to-Site VPN Connections → Create VPN Connection

- Name: hybrid-vpn | Target Gateway Type: Virtual Private Gateway → hybrid-vgw

- Customer Gateway: Existing → ubuntu-strongswan

- Routing Options: Static | Static IP Prefixes: 192.168.91.0/24

- Tunnel 1 Pre-Shared Key: HybridLab2024

- Tunnel 2 Pre-Shared Key: HybridLab2024B

- Create VPN Connection — wait 5-10 minutes for status: Available

- Download Configuration: select VPN → Download Configuration → Vendor: Generic → Download

The downloaded config file contained the following critical values used throughout the lab:

| Parameter | Tunnel 1 | Tunnel 2 |
| --- | --- | --- |
| AWS Outside IP | 52.55.138.58 | 52.206.45.187 |
| Customer Outside IP | 172.220.62.45 | 172.220.62.45 |
| AWS Inside IP | 169.254.46.237/30 | 169.254.54.153/30 |
| Customer Inside IP | 169.254.46.238/30 | 169.254.54.154/30 |
| Pre-Shared Key | HybridLab2024 | HybridLab2024B |
| IKE Version | IKEv1 | IKEv1 |
| IKE Encryption | AES-128-CBC | AES-128-CBC |
| IKE Auth | SHA1 | SHA1 |
| IKE DH Group | Group 2 (modp1024) | Group 2 (modp1024) |
| ESP Encryption | AES-128-CBC | AES-128-CBC |
| ESP Auth | HMAC-SHA1-96 | HMAC-SHA1-96 |
| ESP DH Group | Group 2 (modp1024) | Group 2 (modp1024) |
| IKE Lifetime | 28800 seconds | 28800 seconds |
| ESP Lifetime | 3600 seconds | 3600 seconds |
| MTU | 1436 bytes | 1436 bytes |

## 2.8 Enable Route Propagation

- VPC → Route Tables → select hybrid-private-rt

- Route Propagation tab → Edit Route Propagation → Enable next to hybrid-vgw → Save

Once the VPN tunnel comes UP, AWS automatically injects 192.168.91.0/24 → VGW into this route table without manual intervention.

## 2.9 Security Group

- VPC → Security Groups → Create Security Group

- Name: hybrid-ec2-sg | VPC: hybrid-vpc

| Type | Protocol | Port | Source | Purpose |
| --- | --- | --- | --- | --- |
| All ICMP IPv4 | ICMP | All | 192.168.91.0/24 | Ping from Ubuntu VM |
| SSH | TCP | 22 | 192.168.91.0/24 | SSH from Ubuntu VM over VPN |
| All traffic | All | All | 10.0.0.0/16 | Free communication within VPC |

# 3. Phase 2 — Ubuntu VM Configuration

All commands run on Ubuntu 24.04 LTS in VMware Workstation with the network adapter set to Bridged mode.

## 3.1 VMware Network Adapter

- VMware Workstation → VM Settings → Network Adapter → Bridged (Connected directly to physical network)

- Check: Replicate physical network connection state

- Configure Adapters → ensure your physical NIC is checked

### Verify Ubuntu has correct LAN IP

ip addr show

# Expected: inet 192.168.91.145/24 on ens33

ip route show

# Expected: default via 192.168.91.1

## 3.2 Install strongSwan

sudo apt update && sudo apt upgrade -y

sudo apt install -y \

strongswan \

strongswan-starter \

strongswan-pki \

libcharon-extra-plugins \

libcharon-extauth-plugins \

libstrongswan-extra-plugins \

net-tools iproute2

# Verify installation

ipsec version

# Returns: Linux strongSwan U5.9.13

## 3.3 Enable IP Forwarding

Ubuntu must forward packets between the VPN tunnel interface and the LAN. Without this, traffic from other devices on the network cannot reach AWS.

sudo tee /etc/sysctl.d/99-vpn-hybrid.conf << 'EOF'

net.ipv4.ip_forward = 1

net.ipv4.conf.all.accept_redirects = 0

net.ipv4.conf.all.send_redirects = 0

net.ipv4.conf.default.rp_filter = 0

net.ipv4.conf.all.rp_filter = 0

net.ipv4.conf.default.accept_redirects = 0

net.ipv4.conf.default.send_redirects = 0

EOF

sudo sysctl -p /etc/sysctl.d/99-vpn-hybrid.conf

sysctl net.ipv4.ip_forward   # Must return: 1

## 3.4 Configure Pre-Shared Keys — /etc/ipsec.secrets

sudo bash -c 'cat > /etc/ipsec.secrets << EOF

172.220.62.45 52.55.138.58  : PSK "HybridLab2024"

172.220.62.45 52.206.45.187 : PSK "HybridLab2024B"

EOF'

sudo chmod 600 /etc/ipsec.secrets

sudo chown root:root /etc/ipsec.secrets

## 3.5 Configure /etc/ipsec.conf

The final working configuration after resolving all issues (see Section 5):

sudo bash -c 'cat > /etc/ipsec.conf << EOF

config setup

charondebug="ike 2, knl 2, cfg 2, net 2, esp 2, dmn 2, mgr 2"

uniqueids=no

conn aws-tunnel-1

auto=start

authby=secret

type=tunnel

left=192.168.91.145

leftid=172.220.62.45

leftsubnet=192.168.91.0/24

right=52.55.138.58

rightid=52.55.138.58

rightsubnet=10.0.0.0/16

keyexchange=ikev1

ike=aes128-sha1-modp1024!

ikelifetime=28800s

esp=aes128-sha1-modp1024!

lifetime=3600s

aggressive=no

dpdaction=restart

dpddelay=10s

dpdtimeout=30s

forceencaps=yes

keyingtries=%forever

rekey=yes

mark=100

conn aws-tunnel-2

auto=start

authby=secret

type=tunnel

left=192.168.91.145

leftid=172.220.62.45

leftsubnet=192.168.91.0/24

right=52.206.45.187

rightid=52.206.45.187

rightsubnet=10.0.0.0/16

keyexchange=ikev1

ike=aes128-sha1-modp1024!

ikelifetime=28800s

esp=aes128-sha1-modp1024!

lifetime=3600s

aggressive=no

dpdaction=restart

dpddelay=10s

dpdtimeout=30s

forceencaps=yes

keyingtries=%forever

rekey=yes

mark=200

EOF'

## 3.6 Fix /etc/strongswan.conf

The strongswan.conf must contain exactly this — any extra content causes a fatal syntax error:

sudo tee /etc/strongswan.conf > /dev/null << 'EOF'

charon {

load_modular = yes

plugins {

include strongswan.d/charon/*.conf

}

}

include strongswan.d/*.conf

EOF

## 3.7 Configure Firewall (iptables)

# Allow IKE and NAT-T

sudo ufw allow 500/udp

sudo ufw allow 4500/udp

# Allow ESP protocol

sudo iptables -A INPUT  -p esp -j ACCEPT

sudo iptables -A OUTPUT -p esp -j ACCEPT

sudo iptables -A FORWARD -p esp -j ACCEPT

# Allow VTI interface traffic

sudo iptables -A FORWARD -i vti1 -j ACCEPT

sudo iptables -A FORWARD -o vti1 -j ACCEPT

sudo iptables -A FORWARD -i vti2 -j ACCEPT

sudo iptables -A FORWARD -o vti2 -j ACCEPT

# Masquerade on-prem traffic to AWS

sudo iptables -t nat -A POSTROUTING -s 192.168.91.0/24 \

-d 10.0.0.0/16 -j MASQUERADE

# Save rules (iptables-persistent replaced UFW — see Issue #3)

sudo apt install -y iptables-persistent

sudo netfilter-persistent save

## 3.8 Create VTI Interfaces

Virtual Tunnel Interfaces (VTI) bind the IPSec tunnel to a routable network interface. These must be created manually before the tunnels come up.

# Tunnel 1 VTI

sudo ip tunnel add vti1 \

local 192.168.91.145 \

remote 52.55.138.58 \

mode vti key 100

sudo ip link set vti1 up mtu 1436

sudo ip addr add 169.254.46.238/30 dev vti1

sudo sysctl -w net.ipv4.conf.vti1.disable_policy=1

sudo sysctl -w net.ipv4.conf.vti1.rp_filter=0

# Tunnel 2 VTI

sudo ip tunnel add vti2 \

local 192.168.91.145 \

remote 52.206.45.187 \

mode vti key 200

sudo ip link set vti2 up mtu 1436

sudo ip addr add 169.254.54.154/30 dev vti2

sudo sysctl -w net.ipv4.conf.vti2.disable_policy=1

sudo sysctl -w net.ipv4.conf.vti2.rp_filter=0

# Add routes to AWS VPC via VTI interfaces

sudo ip route add 10.0.0.0/16 dev vti1 metric 100

sudo ip route add 10.0.0.0/16 dev vti2 metric 200

## 3.9 Start strongSwan and Bring Up Tunnels

# Remove stale PID files (if any)

sudo rm -f /var/run/charon.pid /var/run/starter.charon.pid

# Enable and start strongSwan

sudo systemctl enable strongswan-starter

sudo systemctl start strongswan-starter

sleep 3

sudo systemctl status strongswan-starter

# Bring up both tunnels

sudo ipsec up aws-tunnel-1

sudo ipsec up aws-tunnel-2

# Verify

sudo ipsec status

💡 Expected ipsec status output aws-tunnel-1[1]: ESTABLISHED, 192.168.91.145[172.220.62.45]...52.55.138.58 aws-tunnel-1{7}: INSTALLED, TUNNEL, ESP in UDP SPIs: c68c1957_i c1585fa2_o aws-tunnel-1{7}: 192.168.91.0/24 === 10.0.0.0/16 aws-tunnel-2[4]: ESTABLISHED, 192.168.91.145[172.220.62.45]...52.206.45.187 aws-tunnel-2{8}: INSTALLED, TUNNEL, ESP in UDP SPIs: c3269caa_i cb7d589a_o aws-tunnel-2{8}: 192.168.91.0/24 === 10.0.0.0/16

# 4. Phase 3 — EC2 Instance Creation

## 4.1 Create Key Pair

- EC2 → Key Pairs → Create Key Pair

- Name: hybrid-keypair | Type: RSA | Format: .pem

- Create — browser downloads hybrid-keypair.pem

# On Ubuntu VM — set correct permissions

mkdir -p ~/.ssh

mv ~/Downloads/hybrid-keypair.pem ~/.ssh/

chmod 400 ~/.ssh/hybrid-keypair.pem

ls -la ~/.ssh/hybrid-keypair.pem

# Must show: -r-------- (400)

## 4.2 Create IAM Role for SSM

- IAM → Roles → Create Role → Trusted entity: AWS Service → EC2

- Attach permission: AmazonSSMManagedInstanceCore

- Role name: hybrid-ssm-role → Create Role

## 4.3 Launch EC2 Instance

| Setting | Value |
| --- | --- |
| Name | hybrid-cloud-server |
| AMI | Amazon Linux 2023 (free tier) |
| Instance Type | t3.micro |
| Key Pair | hybrid-keypair |
| VPC | hybrid-vpc |
| Subnet | hybrid-private-1a (10.0.1.0/24) |
| Auto-assign Public IP | Disabled (private subnet — no public IP) |
| Security Group | hybrid-ec2-sg |
| IAM Instance Profile | hybrid-ssm-role |

- Click Launch Instance — wait for Instance State: Running and Status Checks: 2/2 passed

- Note the Private IPv4 address — in this lab it was assigned 10.0.1.23

⚠️ Private Subnet — No Public IP The EC2 instance is in a private subnet with no public IP. It is only reachable from the on-prem network via the VPN tunnel. This is the correct hybrid architecture — cloud resources are never publicly exposed.

# 5. Issues Encountered & Resolutions

This section documents every problem encountered during the lab in chronological order. Each issue includes the exact error message, the root cause, and the exact commands used to resolve it.

ISSUE #1 — iptables-persistent Removed UFW

🔴 Symptom: Running: sudo apt install -y iptables-persistent Output: The following packages will be REMOVED: ufw Followed by: sudo: ufw: command not found

🔍 Root Cause: On Ubuntu 24.04 Noble, iptables-persistent conflicts with UFW because they both manage the netfilter kernel firewall. The package manager chose iptables-persistent and removed UFW automatically.

✅ Resolution: This is expected behaviour and not a problem. UFW and iptables-persistent serve the same purpose — managing kernel netfilter rules. Since iptables-persistent was already installed and our rules were saved, we continued without UFW. All firewall rules (ESP, UDP 500, UDP 4500, FORWARD, MASQUERADE) were already applied in the kernel and saved by netfilter-persistent.

Commands Used: sudo netfilter-persistent save # Verify rules saved: sudo cat /etc/iptables/rules.v4

ISSUE #2 — NO_PROPOSAL_CHOSEN on Phase 2 (QUICK_MODE)

🔴 Symptom: sudo ipsec up aws-tunnel-1 generating QUICK_MODE request ... [ HASH SA No ID ID ] received NO_PROPOSAL_CHOSEN error notify establishing connection 'aws-tunnel-1' failed Note: Phase 1 (IKE_SA) was ESTABLISHED — only Phase 2 failed.

🔍 Root Cause: The esp= line in ipsec.conf was set to esp=aes128-sha1! (without the DH group). strongSwan sent a Phase 2 proposal without a DH group. AWS requires DH Group 2 (modp1024) for Phase 2 PFS, so it rejected with NO_PROPOSAL_CHOSEN. The ! suffix forces strict matching, so the proposal had no fallback.

✅ Resolution: Add modp1024 back to the ESP proposal. The correct value matching the AWS config file is: esp=aes128-sha1-modp1024!

Commands Used: sudo sed -i 's/esp=aes128-sha1!/esp=aes128-sha1-modp1024!/g' /etc/ipsec.conf grep 'esp=' /etc/ipsec.conf # Verify both tunnels updated sudo ipsec reload sudo ipsec down aws-tunnel-1 && sudo ipsec up aws-tunnel-1

ISSUE #3 — pfs=no Deprecated Keyword Warning

🔴 Symptom: Starting strongSwan 5.9.13 IPsec [starter]... # deprecated keyword 'pfs' in conn 'aws-tunnel-1' PFS is enabled by specifying a DH group in the 'esp' cipher suite # deprecated keyword 'pfs' in conn 'aws-tunnel-2' ### 2 parsing errors (0 fatal) ###

🔍 Root Cause: strongSwan 5.9.13 removed the standalone pfs= keyword. PFS (Perfect Forward Secrecy) is now controlled entirely by whether a DH group is included in the esp= cipher suite. The old pfs=no keyword causes a parse warning.

✅ Resolution: Remove pfs=no from both conn blocks in ipsec.conf. PFS behaviour is now controlled by the DH group in esp=. Since we use esp=aes128-sha1-modp1024!, PFS is enabled via modp1024.

Commands Used: # Remove pfs=no from ipsec.conf sudo sed -i '/pfs=no/d' /etc/ipsec.conf grep -n 'pfs' /etc/ipsec.conf # Must return nothing

ISSUE #4 — /etc/strongswan.conf Syntax Error — Unclosed Brace

🔴 Symptom: sudo systemctl start strongswan-starter Active: inactive (dead) ipsec[xxx]: 00[CFG] syntax error, unexpected end of file, expecting NAME or NEWLINE or '}' ipsec[xxx]: 00[CFG] invalid config file '/etc/strongswan.conf' ipsec[xxx]: 00[LIB] abort initialization due to invalid configuration charon has quit: integrity test of libstrongswan failed

🔍 Root Cause: During an earlier troubleshooting step, the following block was appended to /etc/strongswan.conf using sudo tee -a: charon { i_dont_care_about_security... = no cisco_unity = yes close_ike_on_child_failure = yes } This created a SECOND charon { block inside the file. The file now had two charon { blocks — the original one with its own nested plugins { } and the new one. The parser hit end-of-file expecting to close a brace from the outer structure.

✅ Resolution: Completely overwrite strongswan.conf with the minimal correct content using tee (not tee -a) to prevent any appending.

Commands Used: sudo tee /etc/strongswan.conf > /dev/null << 'EOF' charon { load_modular = yes plugins { include strongswan.d/charon/*.conf } } include strongswan.d/*.conf EOF # Verify brace balance python3 -c " c = open('/etc/strongswan.conf').read() o, cl = c.count('{'), c.count('}') print('OK' if o == cl else 'BROKEN')"

ISSUE #5 — /etc/strongswan.d/charon.conf Corrupted — Broken Block Prepended

🔴 Symptom: Even after fixing strongswan.conf, strongSwan still failed with the SAME syntax error: syntax error, unexpected end of file, expecting NAME or NEWLINE or '}' invalid config file '/etc/strongswan.conf' But strongswan.conf was clean! The error was coming from an included file.

🔍 Root Cause: The file /etc/strongswan.d/charon.conf was corrupted. An earlier command used: sudo tee -a /etc/strongswan.conf but it actually appended to charon.conf instead (shell redirection error). The prepended block was at lines 1-5: Line 1: # AWS VPN compatibility — allow DH Group 2 Line 3: i_dont_care_about_security_and_use_aggressive_mode_psk = no Line 4: cisco_unity = yes Line 5: close_ike_on_child_failure = yes These lines had no opening charon { to match the content, so the parser found content before the expected block opening.

✅ Resolution: Identify the exact lines using grep -n, then use sed -i to delete exactly those lines from the top of charon.conf.

Commands Used: # Find the bad lines grep -n 'AWS VPN compatibility¦cisco_unity¦i_dont_care' \ /etc/strongswan.d/charon.conf # Returned: lines 1-5 were our bad prepend # Delete lines 1-5 sudo sed -i '1,5d' /etc/strongswan.d/charon.conf # Verify braces balanced python3 -c " c = open('/etc/strongswan.d/charon.conf').read() o, cl = c.count('{'), c.count('}') print(f'Opens: {o} Closes: {cl}') print('FIXED' if o == cl else 'STILL BROKEN')"

ISSUE #6 — Stale PID Files Blocking strongSwan Restart

🔴 Symptom: sudo systemctl start strongswan-starter Output: charon is already running (/var/run/charon.pid exists) -- skipping daemon start starter is already running (/var/run/starter.charon.pid exists) -- no fork done But ps aux showed no charon or starter processes running.

🔍 Root Cause: When strongSwan was killed with pkill -9, it didn't get a chance to clean up its PID files at /var/run/charon.pid and /var/run/starter.charon.pid. On the next start attempt, the starter script found these files and assumed the daemon was still running — even though it wasn't.

✅ Resolution: Manually remove the stale PID files, then start the service normally.

Commands Used: sudo rm -f /var/run/charon.pid sudo rm -f /var/run/starter.charon.pid sudo systemctl start strongswan-starter sudo systemctl status strongswan-starter

ISSUE #7 — VTI Interfaces Not Auto-Created

🔴 Symptom: After strongSwan started and tunnels showed ESTABLISHED, running: ip addr show vti1 Returned: Device "vti1" does not exist. With no VTI interfaces, there were no routes to 10.0.0.0/16 and ip xfrm policy was empty.

🔍 Root Cause: The updown script approach for automatic VTI creation was not working correctly in this environment. The strongSwan VTI updown hook requires specific configuration and the script was not being called on tunnel establishment.

✅ Resolution: Create the VTI interfaces and routes manually. This is reliable and can be made persistent via /etc/networkd-dispatcher.

Commands Used: sudo ip tunnel add vti1 local 192.168.91.145 remote 52.55.138.58 mode vti key 100 sudo ip link set vti1 up mtu 1436 sudo ip addr add 169.254.46.238/30 dev vti1 sudo sysctl -w net.ipv4.conf.vti1.disable_policy=1 sudo sysctl -w net.ipv4.conf.vti1.rp_filter=0 sudo ip tunnel add vti2 local 192.168.91.145 remote 52.206.45.187 mode vti key 200 sudo ip link set vti2 up mtu 1436 sudo ip addr add 169.254.54.154/30 dev vti2 sudo sysctl -w net.ipv4.conf.vti2.disable_policy=1 sudo sysctl -w net.ipv4.conf.vti2.rp_filter=0 sudo ip route add 10.0.0.0/16 dev vti1 metric 100 sudo ip route add 10.0.0.0/16 dev vti2 metric 200

ISSUE #8 — strongswan checkconfig and ipsec checkconfig Not Available

🔴 Symptom: sudo strongswan checkconfig sudo: strongswan: command not found sudo ipsec checkconfig /usr/sbin/ipsec: unknown command 'checkconfig'

🔍 Root Cause: strongSwan 5.9.13 on Ubuntu 24.04 does not include the checkconfig subcommand. This was available in older versions but was removed. The ipsec binary only exposes a limited set of commands on this version.

✅ Resolution: Use these alternatives to validate config files before starting strongSwan: 1. Run starter in foreground to catch errors: sudo /usr/lib/ipsec/starter --nofork 2. Python brace balance check for strongswan.conf and charon.conf 3. Simply attempt to start and read systemctl status output

Commands Used: # Test config by running starter directly sudo /usr/lib/ipsec/starter --nofork 2>&1 ¦ head -10 # Ctrl+C to stop after checking for errors # Python brace checker python3 -c " import glob for f in ['/etc/strongswan.conf'] + glob.glob('/etc/strongswan.d/*.conf'): c = open(f).read() o, cl = c.count('{'), c.count('}') if o != cl: print(f'BROKEN: {f} opens={o} closes={cl}')"

# 6. Final Verification Results

All tests were run after resolving all issues. The following results confirm full bidirectional connectivity between the Ubuntu VM and the AWS EC2 instance over the IPSec VPN tunnel.

## 6.1 VPN Tunnel Status

Both IPSec tunnels showed ESTABLISHED and INSTALLED — Phase 1 and Phase 2 both successful:

Security Associations (2 up, 0 connecting):

aws-tunnel-2[4]: ESTABLISHED 17 minutes ago,

192.168.91.145[172.220.62.45]...52.206.45.187[52.206.45.187]

aws-tunnel-2{8}: INSTALLED, TUNNEL, ESP in UDP SPIs: c3269caa_i cb7d589a_o

aws-tunnel-2{8}: 192.168.91.0/24 === 10.0.0.0/16

aws-tunnel-1[3]: ESTABLISHED 17 minutes ago,

192.168.91.145[172.220.62.45]...52.55.138.58[52.55.138.58]

aws-tunnel-1{7}: INSTALLED, TUNNEL, ESP in UDP SPIs: c68c1957_i c1585fa2_o

aws-tunnel-1{7}: 192.168.91.0/24 === 10.0.0.0/16

## 6.2 AWS Console Tunnel Status

AWS Console → VPC → Site-to-Site VPN Connections → vpn-0a9aa2a54ab7106e3 → Tunnel Details:

| Tunnel | Outside IP | Inside IPv4 CIDR | Status | Provisioning |
| --- | --- | --- | --- | --- |
| Tunnel 1 | 52.55.138.58 | 169.254.46.236/30 | ✅ Up | ✅ Available |
| Tunnel 2 | 52.206.45.187 | 169.254.54.152/30 | ✅ Up | ✅ Available |

## 6.3 Routing Table

ip route show | grep '10.0'

10.0.0.0/16 dev vti1 scope link metric 100

10.0.0.0/16 dev vti2 scope link metric 200

## 6.4 Ping Test — Ubuntu VM to EC2

ping -c 3 10.0.1.23

PING 10.0.1.23 (10.0.1.23) 56(84) bytes of data.

64 bytes from 10.0.1.23: icmp_seq=1 ttl=127 time=51.4 ms

64 bytes from 10.0.1.23: icmp_seq=2 ttl=127 time=52.2 ms

64 bytes from 10.0.1.23: icmp_seq=3 ttl=127 time=50.4 ms

--- 10.0.1.23 ping statistics ---

3 packets transmitted, 3 received, 0% packet loss, time 2004ms

rtt min/avg/max/mdev = 50.409/51.315/52.170/0.719 ms

🎉 Ping Successful 3/3 packets received, 0% packet loss. Average RTT ~51ms confirms encrypted traffic is flowing through the IPSec tunnel from Ubuntu VM (192.168.91.145) to AWS EC2 (10.0.1.23).

## 6.5 TCP Port 22 (SSH) Test

nc -zv 10.0.1.23 22 2>&1

Connection to 10.0.1.23 22 port [tcp/ssh] succeeded!

## 6.6 SSH Connection — Ubuntu VM to EC2

ssh -i ~/.ssh/hybrid-keypair.pem ec2-user@10.0.1.23

,     #_

~\_  ####_        Amazon Linux 2023

~~  \_#####\

~~     \###|       https://aws.amazon.com/linux/amazon-linux-2023

~~       \#/ ___

~~       V~' '->

~~~         /

~~._.   _/

_/ _/

_/m/'

Last login: Tue Apr  7 20:05:53 2026 from 192.168.91.145

[ec2-user@ip-10-0-1-23 ~]$ hostname

ip-10-0-1-23.ec2.internal

🎉 SSH Successful Successfully authenticated and connected to the private EC2 instance (10.0.1.23) from the Ubuntu VM (192.168.91.145) over the VPN tunnel — using a private key. The EC2 instance has NO public IP and is only accessible via the VPN.

# 7. Quick Reference

## 7.1 Key Commands

| Task | Command |
| --- | --- |
| Check tunnel status | sudo ipsec status |
| Bring up tunnel 1 | sudo ipsec up aws-tunnel-1 |
| Bring up tunnel 2 | sudo ipsec up aws-tunnel-2 |
| Bring down tunnel | sudo ipsec down aws-tunnel-1 |
| Reload config | sudo ipsec reload |
| Restart strongSwan | sudo systemctl restart strongswan-starter |
| View live logs | sudo journalctl -fu strongswan-starter |
| Check VTI interfaces | ip addr show vti1 && ip addr show vti2 |
| Check routes to AWS | ip route show ¦ grep '10.0' |
| Check kernel policies | sudo ip xfrm policy |
| Ping EC2 | ping -c 4 10.0.1.23 |
| SSH to EC2 | ssh -i ~/.ssh/hybrid-keypair.pem ec2-user@10.0.1.23 |
| Check IP forwarding | sysctl net.ipv4.ip_forward |
| Verify brace balance | python3 -c "c=open('/etc/strongswan.conf').read(); print('OK' if c.count('{')==c.count('}') else 'BROKEN')" |

## 7.2 Troubleshooting Reference

| Error | Cause | Fix |
| --- | --- | --- |
| NO_PROPOSAL_CHOSEN | ESP DH group mismatch | esp=aes128-sha1-modp1024! (add modp1024) |
| syntax error unexpected end of file | Unclosed { in strongswan.conf or charon.conf | Check brace balance; fix or restore the broken file |
| charon.pid exists — skipping | Stale PID files from killed process | rm -f /var/run/charon.pid /var/run/starter.charon.pid |
| AUTHENTICATION_FAILED | PSK mismatch | Check /etc/ipsec.secrets — copy PSK exactly |
| ESTABLISHED but no INSTALLED | VTI interfaces missing or route missing | Manually create vti1/vti2; add ip route |
| Ping fails — tunnel INSTALLED | Security group blocks ICMP | Add All ICMP from 192.168.91.0/24 to hybrid-ec2-sg |
| strongswan: command not found | Binary name differs on this distro | Use ipsec directly: sudo ipsec up/down/status/reload |

## 7.3 Lab Completion Checklist

| Item | Status |
| --- | --- |
| VPC 10.0.0.0/16 created | ✅ Complete |
| Public and private subnets created | ✅ Complete |
| Virtual Private Gateway attached | ✅ Complete |
| Customer Gateway 172.220.62.45 created | ✅ Complete |
| VPN Connection vpn-0a9aa2a54ab7106e3 created | ✅ Complete |
| Route propagation enabled on private RT | ✅ Complete |
| strongSwan installed and configured | ✅ Complete |
| ipsec.conf with correct AES/SHA1/modp1024 | ✅ Complete |
| ipsec.secrets with correct PSKs | ✅ Complete |
| strongswan.conf clean (2 opens, 2 closes) | ✅ Complete |
| charon.conf restored (lines 1-5 removed) | ✅ Complete |
| VTI interfaces vti1 and vti2 created | ✅ Complete |
| Routes 10.0.0.0/16 via vti1/vti2 | ✅ Complete |
| Tunnel 1 ESTABLISHED + INSTALLED | ✅ Complete |
| Tunnel 2 ESTABLISHED + INSTALLED | ✅ Complete |
| AWS Console — Tunnel 1 UP | ✅ Complete |
| AWS Console — Tunnel 2 UP | ✅ Complete |
| EC2 instance created in private subnet | ✅ Complete |
| Ping 10.0.1.23 — 0% packet loss | ✅ Complete |
| SSH to ec2-user@10.0.1.23 — Success | ✅ Complete |